QUAD

Qualcomm Unified Agent for Developers

Sample App Prompts

Copy-paste prompts for Claude Code + QUAD MCP Server
Works in mock mode — no hardware or SDK required

Quick Start: git clone <repo> && cd QUAD && ./setup.sh && source .venv/bin/activate

# Contents

1.  Getting Started  —  How QUAD works with Claude Code

2.  Hardware Detection  —  Discover chipset, CPU, GPU, NPU specs

3.  Model Conversion  —  Convert ONNX/PyTorch/TF to DLC

4.  Profiling  —  Latency, power, HTP linting, QHAS

5.  Workload Orchestration  —  Allocate layers across CPU/GPU/NPU

6.  Code Generation  —  Generate C++, Python, Kotlin

7.  Full End-to-End Workflows  —  Multi-step pipeline prompts

8.  Multi-Model & Platform Comparison  —  Compare models side-by-side

9.  Optimization Workflows  —  Find and fix HTP bottlenecks

10.  Advanced Scenarios  —  Power modes, quantization, QHAS

# 1. Getting Started

QUAD exposes 5 MCP tools that Claude Code can call: hardware_detect, convert_model, profile_workload, orchestrate_workload, and generate_code. Once the server is running, just describe what you want — Claude calls the right tools automatically.

All prompts below work in mock mode — no Qualcomm hardware or SDK installation needed.

Start the MCP server:

💬  Terminal

quad-server --config quad.toml

Open this project in Claude Code — QUAD tools are auto-registered.

💡  Tip: Use 'mock mode' for all development and demos. Switch to 'real' mode only when the QNN/SNPE SDK is installed.

────────────────────────────────────────────────────────────────────────

# 2. Hardware Detection

Detect the chipset, CPU, GPU, and NPU topology for any Qualcomm platform.

Detect Windows AI PC (Snapdragon X Elite):

💬  Prompt

Use QUAD to detect the hardware on my Windows AI PC. Show the chipset name, CPU cores, GPU TFLOPS, and NPU TOPS.

📦  What QUAD returns

• Chipset: Snapdragon X Elite X1E-80-100 • CPU: 12 × Oryon ARM64 @ 3.8 GHz • GPU: Adreno X1-85 (4.6 TFLOPS) • NPU: Hexagon NPU (45.0 TOPS) • Available runtimes: cpu, gpu, npu

Detect Android device (Snapdragon 8 Elite):

💬  Prompt

Detect hardware for the Android platform using QUAD. What NPU is available and how many TOPS does it have?

📦  What QUAD returns

• Chipset: Snapdragon 8 Elite SM8750 • NPU: Hexagon NPU (HTP) — 48.0 TOPS • GPU: Adreno 830 (5.0 TFLOPS)

Detect Linux embedded (QCS2210):

💬  Prompt

Use QUAD hardware_detect for the linux platform. Show all compute resources available.

📦  What QUAD returns

• Chipset: QCS2210 (Qualcomm Robotics RB1) • CPU: 4 × Kryo ARM64 @ 2.0 GHz • NPU: Hexagon DSP V66 (1.0 TOPS) • RAM: 2 GB

────────────────────────────────────────────────────────────────────────

# 3. Model Conversion

Convert ONNX, PyTorch, TensorFlow, or TFLite models to SNPE DLC or QNN format.

Basic conversion — INT8 quantization:

💬  Prompt

Use QUAD to convert resnet50.onnx to SNPE format with INT8 quantization for Windows. Show the output file, compression ratio, and any image format requirements.

📦  What QUAD returns

• Output: resnet50.dlc • Original: 25.0 MB → Converted: 6.2 MB (4.0× compression) • Quantization: int8 applied • Image format note: NCHW → NHWC transpose needed before inference

PyTorch model with BGR channel order (e.g. AlexNet):

💬  Prompt

Convert alexnet.onnx to SNPE DLC. The model was trained with BGR channel order and PyTorch NCHW layout. Show me exactly how to prepare input images.

📦  What QUAD returns

• channel_order: bgr — ensure input images are in BGR order • input_layout: nchw — transpose: np.transpose(img, (0,2,3,1)) → NHWC • Output: alexnet.dlc

MobilenetSSD object detection model:

💬  Prompt

Convert ssd_mobilenet_v2_quantized_300x300.pb (TensorFlow frozen graph) to SNPE DLC for GPU inference. Show any special requirements for this model.

📦  What QUAD returns

• Conversion notes: MobilenetSSD requires allow_unconsumed_nodes=True • DetectionOutput layer: CPU only — enable CPU fallback for GPU/DSP • Output nodes: detection_classes, detection_boxes, detection_scores

💡  Tip: Always check conversion_notes in the result — QUAD surfaces model-specific tips automatically.

────────────────────────────────────────────────────────────────────────

# 4. Profiling

Profile inference performance at multiple levels: timing, HTP cycle counts, or full QHAS analysis.

Standard detailed profiling:

💬  Prompt

Profile resnet50.dlc on the Windows NPU using QUAD. Show mean latency, p95 latency, throughput, power consumption, and memory usage.

📦  What QUAD returns

• Latency: mean=5.0ms p95=7.0ms p99=9.0ms • Throughput: 200 FPS • Power: 2000 mW • Memory: peak=45 MB avg=38 MB • NPU utilization: 89%

HTP Linting profile — cycle-based bottleneck analysis:

💬  Prompt

Use QUAD to profile mobilenetv2.dlc with HTP linting mode. Identify the top bottleneck ops and explain how to fix them.

📦  What QUAD returns

• Total cycles: 3,753,939 • Bottleneck: sub_op (2,165,162 cycles, 21% overlap) ⚠️ • Hint: Replace Sub with Conv for better HTP parallelism (~68% improvement) • Well-parallelized: add_op (92% overlap) ✅

QHAS profiling — full QNN HTP Analysis Summary:

💬  Prompt

Run QHAS profiling on yolov8n.dlc using QUAD. What chrometrace file does it generate and how do I view it?

📦  What QUAD returns

• Runs 3-step pipeline: graph-prepare → net-run → profile-viewer • Generates: chrometrace.json (open in chrome://tracing) • Also produces: _htp.json with op-by-op topology

💡  Tip: Use 'detailed' for latency/power numbers. Use 'linting' to find HTP bottlenecks. Use 'qhas' for deep analysis with chrome tracing.

────────────────────────────────────────────────────────────────────────

# 5. Workload Orchestration

Automatically allocate model layers across CPU, GPU, and NPU for a given power budget.

Performance mode — maximize NPU usage:

💬  Prompt

Use QUAD to orchestrate resnet50.dlc on Windows in performance mode. Show which layers go to NPU vs CPU and the projected latency.

📦  What QUAD returns

• NPU utilization: 70% ¦ CPU: 30% ¦ GPU: 0% • Projected latency: 2.55 ms • Projected power: 1420 mW • CPU fallback ops: BatchNorm layers (unsupported on HTP)

Compare all three power modes:

💬  Prompt

Using QUAD, compare performance, balanced, and efficiency power modes for mobilenetv2.dlc on Android. Show latency and power for each.

📦  What QUAD returns

• performance → 2.55ms 1420mW NPU=70% • balanced → 2.55ms 1420mW NPU=70% • efficiency → 5.00ms 1000mW NPU=0% (CPU only)

Low-power embedded (QCS2210, 3W budget):

💬  Prompt

Orchestrate yolov8n.dlc for the linux platform in efficiency mode. The device has a 3W power budget. What's the recommended allocation?

────────────────────────────────────────────────────────────────────────

# 6. Code Generation

Generate production-ready inference code for C++, Python, Kotlin, or Arduino.

C++ inference app for Windows AI PC:

💬  Prompt

Use QUAD to generate a C++ inference application for resnet50.dlc on Windows using the QNN SDK. Include the CMakeLists.txt build file.

📦  What QUAD returns

• inference.cpp — full QNN initialization, graph execute, output read • CMakeLists.txt — build config targeting Windows ARM64 • Build: mkdir build && cd build && cmake .. && cmake --build .

Python inference script for Android:

💬  Prompt

Generate Python inference code for mobilenetv2.dlc on Android using SNPE. I need to run it via ADB.

Kotlin Android AAR for mobile app:

💬  Prompt

Use QUAD generate_code for the Android platform with Kotlin language for yolov8n.dlc. Generate an Android inference module I can add to my app.

Arduino sketch for embedded Linux:

💬  Prompt

Generate an Arduino-compatible inference sketch for the linux platform using SNPE SDK for a simple classification model.

────────────────────────────────────────────────────────────────────────

# 7. Full End-to-End Workflows

Use multiple QUAD tools together in a single conversation flow.

ResNet-50 on Windows AI PC — full pipeline:

💬  Prompt

Build a complete inference pipeline for resnet50.onnx on Windows AI PC using QUAD: 1. Detect the hardware 2. Convert to SNPE DLC with INT8 quantization 3. Profile with detailed timing 4. Profile with HTP linting to find bottlenecks 5. Orchestrate in balanced mode 6. Generate C++ inference code Save everything to examples/resnet50_windows.py

MobileNetV2 on Android — TTFI under 5 seconds:

💬  Prompt

Using QUAD, run the full workflow for mobilenetv2.onnx on Android. I need Time-to-First-Inference under 5 seconds. Use INT8 quantization and balanced power mode. Generate Kotlin code at the end.

YOLOv8 object detection on all 3 platforms:

💬  Prompt

Use QUAD to run the full pipeline for yolov8n.onnx across all three platforms: Windows AI PC, Android, and Linux/QCS2210. Compare the latency and NPU utilization for each. Generate Python code for each platform.

────────────────────────────────────────────────────────────────────────

# 8. Multi-Model & Platform Comparison

Use QUAD to benchmark and compare multiple models side-by-side.

Latency comparison across models:

💬  Prompt

Use QUAD to compare inference latency for three models on Windows AI PC: mobilenetv2.onnx, resnet50.onnx, and yolov8n.onnx. Use INT8 quantization and NPU runtime for all. Present results in a table showing latency, throughput, and power.

Platform suitability analysis:

💬  Prompt

I have mobilenetv2.onnx. Use QUAD to help me decide which Qualcomm platform is best for my use case — Windows AI PC, Android, or embedded Linux. Compare NPU TOPS, expected latency, and power consumption.

SDK comparison — QNN vs SNPE:

💬  Prompt

Use QUAD to convert resnet50.onnx twice — once to QNN format and once to SNPE DLC. Compare the output file sizes and tell me which SDK is better for my Windows AI PC use case.

────────────────────────────────────────────────────────────────────────

# 9. Optimization Workflows

Use QUAD linting and architecture analysis to identify and fix performance bottlenecks.

Find and fix HTP bottlenecks:

💬  Prompt

Profile my_model.dlc with QUAD linting mode. For each bottleneck op found (low overlap %), explain why it's slow and what op substitution I should make to improve HTP parallelism.

📦  What QUAD returns

• Sub op → replace with Conv2D (designed weights): −68% cycles • Div op → replace with Mul (reciprocal): −65% cycles • PReLU → replace with ReLU: better HTP parallelism

Architecture checker analysis:

💬  Prompt

Run QUAD's architecture checker workflow on my_model.dlc. Show all HTP performance issues and apply the recommended modifications.

📦  What QUAD returns

• Issue: 16-bit activation → recommend 8-bit for better memory efficiency • Issue: Conv channel < 32 → increase to 32+ for better HTP utilization • Issue: ElementWiseDivide → replace with ElementWiseMultiply • Modifications applied via --modify apply=elwisediv

Quantization impact analysis:

💬  Prompt

Use QUAD to compare the performance impact of FP32 vs INT8 vs INT4 quantization for resnet50.onnx on the Snapdragon X Elite NPU. Show latency, memory, and compression ratio for each.

────────────────────────────────────────────────────────────────────────

# 10. Advanced Scenarios

QHAS chrometrace generation:

💬  Prompt

Use QUAD to run QHAS profiling on mobilenetv2.dlc for SM8750. Generate a chrometrace JSON I can open in chrome://tracing. Enable all features: input/output flow events, runtrace, memory_info.

MobilenetSSD benchmarking setup:

💬  Prompt

Use QUAD to set up a MobilenetSSD benchmark run. Generate the benchmark config JSON, input list file with the correct output layer header, and the snpe_bench.py command to run it.

📦  What QUAD returns

• mobilenetssd_bench.json with CpuFallback=true and BufferTypes=[ub_float, ub_tf8] • imagelist.txt with header: #Postprocessor/BatchMultiClassNonMaxSuppression add_6 • Command: python3 snpe_bench.py -c mobilenetssd_bench.json -a --generate_json

Custom UDO (User-Defined Operation) scaffolding:

💬  Prompt

I need a custom Softmax operation for the HTP backend. Use QUAD to generate the UDO package scaffold — config JSON, DSP implementation stub, and registration library.

Multi-input model with named tensors:

💬  Prompt

Convert my two-input ONNX model to SNPE DLC. The inputs are: 'image' with shape 1,3,224,224 (NCHW, RGB) and 'mask' with shape 1,1,224,224. The output tensor is 'predictions'. Show the exact conversion command.

Accuracy debugging workflow:

💬  Prompt

My SNPE model gives different results from the original ONNX model. Use QUAD to set up an accuracy debugger run: run framework_runner on the ONNX, run inference_engine on the DLC, and compare using CosineSimilarity. Show the verification command.

# Quick Reference — Tool Parameters

| Parameter | Options | Notes |
| --- | --- | --- |
| platform | windows android linux | Selects mock device profile |
| source_format | onnx pytorch tensorflow tflite | Model framework |
| target_sdk | snpe qnn | Output format |
| quantization | fp32 int8 int4 | int8 recommended for NPU |
| input_layout | nchw nhwc auto | PyTorch = nchw |
| channel_order | rgb bgr auto | Caffe/legacy = bgr |
| profiling_level | basic detailed linting qhas | linting = HTP cycles |
| power_mode | performance balanced efficiency | Controls NPU vs CPU split |
| language | cpp python kotlin arduino_sketch | Code gen target |
| runtime | cpu gpu npu auto | Inference runtime |

QUAD — Qualcomm Unified Agent for Developers  |  All prompts work in mock mode without hardware or SDK  |  github.qualcomm.com/pavanr/QUAD

# Appendix — IoT Device Support

This section was added 2026-05-09 to record the dependencies QUAD needs to extend support across Qualcomm's IoT SoC portfolio. It complements (but does not replace) the per-component workbook at docs/IOT_DEPENDENCIES.xlsx.

## Scope

QUAD's adapter pattern is being extended from AI PC and Mobile to the full Qualcomm IoT SoC line. The full dependency catalogue (111 components across 14 categories) is published as a workbook at docs/IOT_DEPENDENCIES.xlsx. This appendix is a quick orientation; the workbook is authoritative.

## Target hardware

| Class | SoC / Board | Where it fits |
| --- | --- | --- |
| Edge gateway | QCS6490 (RB3 Gen 2), QCS8250 (RB5) | Linux-class IoT, 12+ TOPS NPU |
| High-perf IoT / AI | QCS8550 | Industrial AI, smart camera |
| Camera / vision IoT | QCS610 / QCS605 | Smart camera, AI vision |
| Entry IoT | QCM2290 / QCS2290 | Wearables, smart speakers |
| Cellular IoT | Snapdragon X75 / 9205 modem | 5G / NB-IoT / LTE-M uplink |

## Software dependency layers

| Layer | Key dependencies |
| --- | --- |
| OS / Firmware | Yocto meta-qcom, Qualcomm Linux, Zephyr / FreeRTOS, U-Boot, TF-A, OP-TEE |
| Connectivity | BlueZ, OpenThread, Matter 1.4, hostapd, ModemManager + libqmi/libmbim |
| IoT protocols | MQTT (Mosquitto / paho), CoAP (libcoap / aiocoap), LwM2M (Anjay), OPC-UA |
| Cloud + OTA | AWS IoT Device SDK v2, azure-iot-device, Greengrass, IoT Edge, Mender, RAUC, SWUpdate |
| Security | Qualcomm QTEE / SPU, OP-TEE, mbedTLS, OpenSSL, TF-M, PKCS#11, Matter Attestation |
| Edge AI runtime | QNN / QAIRT 2.x, SNPE 2.x, Hexagon SDK 5.x, AIMET, Qualcomm AI Hub |
| Sensors / HAL | libgpiod, smbus2, spidev, pyserial, python-can, pymodbus, bleak |
| Telemetry | OpenTelemetry, Prometheus client_python, Fluent Bit |

## Sample prompts (Claude Code + QUAD MCP)

Use QUAD to detect the Qualcomm IoT SoC on the connected RB3 Gen 2 board and list its CPU/GPU/NPU topology.

Use QUAD to convert mobilenet_v2.onnx to QNN INT8 and profile it on a QCS6490, targeting < 3W power draw.

Use QUAD to allocate a YOLOv8n inference graph across the Hexagon NPU + CPU on QCS8550 within a 5W power budget.

Use QUAD to generate C++ inference code for a Yocto-based RB3 Gen 2 image with QNN runtime + Mender OTA update hooks.

Use QUAD to compare INT8 inference of resnet50.onnx between QCS6490 (gateway) and QCS610 (smart camera) and produce a power-vs-latency table.

## Reference

docs/IOT_DEPENDENCIES.xlsx — full per-component catalogue (priority, license, target version, source URL)

docs/PREREQUISITES.md — base prerequisites for the QUAD agent

QUAD_Server_Guide.docx — section 19 (IoT Device Support)

# Appendix — Real-mode Measurement Fidelity

Added 2026-05-10. Records the real-mode measurement plumbing that landed across the recent QUAD changes (QAIRT-adapter stdout parsers, RSS sampler, host power model, QPM3 / Snapdragon Profiler integration, model registry).

## Provenance-tagged metrics

QUAD's profile_workload tool now returns a measurement_notes block that tags every metric with its provenance: measured (from snpe-diagview / QPM3 / psutil), estimated (host thermal model), or not_measured (with a reason). No fictional defaults — what you see is what was actually captured.

| Metric | Measured source | Fallback when tool absent |
| --- | --- | --- |
| Latency | snpe-diagview CSV (Total Inference Time) | snpe-net-run stdout parser |
| Per-layer | snpe-diagview CSV (Model Layer Times) | synthetic composite (1 row, tagged) |
| Memory RSS | psutil sampler (100 ms polling) | not_measured:rss_unavailable |
| Power | QPM3 (per-frame PMIC readings) | host_thermal_model (CPU/NPU% × TDP) |
| NPU util | Arithmetic: cycles / wall_time / clock | 0.0 (tagged not_measured) |
| GPU util | Snapdragon Profiler chrometrace events | 0.0 (tagged not_measured) |
| CPU util | psutil.cpu_percent | Always available |

## Auto-detected precision profilers

| Tool | Activates when present | Provides |
| --- | --- | --- |
| QPM3 (Qualcomm Power Monitor 3) | PATH or QPM3_HOME env var | Measured per-frame power; flips power tag from estimated to measured |
| Snapdragon Profiler (sdptrace) | PATH or SNAPDRAGON_PROFILER_HOME env var | Real GPU% from chrometrace; populates utilization['gpu'] |
| psutil (always present in [real]) | automatic | RSS sampling + CPU% percent |
| snpe-diagview (in QAIRT) | automatic | Authoritative latency + per-layer; raises tag from snpe-net-run stdout |

## Model registry — `quad models`

Production ONNX provisioning lives in src/quad/model_registry/. Entries declare either a url (auto-downloadable, SHA-256 verified) or a path_env_var (user-supplied for gated weights like Llama 3 8B). Adding a model for a future plan is a single YAML edit — no Python changes needed.

| Command | What it does |
| --- | --- |
| quad models list | Show every registry entry with cache state |
| quad models fetch <name> | Download + verify SHA-256 (URL entries) |
| quad models path <name> | Resolve to local path (env-var entries) |
| quad models verify <name> | Re-check SHA-256 of a cached file |

## Snapdragon X Elite environment caveat

QAIRT 2.46's host Python tools (qairt-converter, qairt-quantizer) ship as windows-arm64ec/.pyd modules that need the full Visual Studio 2022 runtime — not just the VC++ redist, and not native ARM64 Python alone (ARM64EC .pyd can't load into pure ARM64 Python; WinError 193). On Snapdragon X Elite, either install VS 2022 Community (winget install Microsoft.VisualStudio.2022.Community) or run model conversion on a separate x86_64 host and copy the .dlc back. QUAD's runtime path (snpe-net-run, profiling) works on a stock install — only model conversion is affected.

## Reference

src/quad/adapters/parsers.py — pure-function parsers for snpe-net-run, snpe-diagview, qnn-platform-validator, qairt-converter

src/quad/profiler/qpm3.py — Qualcomm Power Monitor 3 wrapper

src/quad/profiler/sdptrace.py — Snapdragon Profiler trace wrapper

src/quad/profiler/rss_sampler.py — psutil-based async RSS sampler

src/quad/profiler/host_power.py — host-side power estimate + SRUM Energy Estimation reader

src/quad/profiler/host_utilization.py — CPU% + NPU/GPU utilisation helpers

src/quad/model_registry/ — production ONNX manifest + fetcher + CLI